*Category (please select) Oral Presenter   Poster presenter  Author(I am submitting an abstract temporarily, I will send a full paper subsequently, which needs to be published in this conference)

Coordinated Emergency Supply Distribution and Multi-Type Personnel Evacuation Optimization for Hazardous Material Incidents on Urban Expressways: A Case Study of the Beijing Sixth Ring Road

Liang Wang1,2  Yu Guan2  He Wei2  Xiaodong Zhang2,*

1 School of Economics and Management, Beijing Jiaotong University, Beijing, 100044, China.

2 Beijing Municipal Institute of City Planning and Design, Beijing, 100045, China.

Liang Wang : wljean@126.com

* Corresponding author: Xiaodong Zhang(zhang-xd23@mails.tsinghua.edu.cn)

Abstract. This study addresses the integrated optimization of emergency supply distribution and multi-type personnel evacuation following a hazardous material (HAZMAT) vehicle accident on the Beijing Sixth Ring Road. A Gaussian plume model simulates chlorine gas dispersion under varying wind conditions, delineating danger zones based on ERPG toxicity thresholds. For supply distribution, a three-tier logistics network with truck–drone coordination is constructed. Supplies are categorized into four priority classes with differentiated time windows (life rescue: 30-min hard window; medical: 60-min; protection: 120-min; basic: 240-min). Results show all 40 tasks completed within deadlines (makespan=44.6 min, zero penalty). Direct truck delivery dominates, indicating sufficient fire station coverage for this incident scale, while drone relay serves as contingency for larger disruptions. For evacuation, a time-stepping model classifies ≈207,000 affected persons into three groups: critical injuries (hospital-mandatory), vulnerable populations (elderly/children), and general evacuees. Five transport modes (ambulances, helicopters, buses, cars, subway) are coordinated. The baseline achieves 25.1% evacuation within 180 minutes, with critical patients fully prioritized. Subway contributes the largest capacity gain (+9.5 points), while helicopter removal disproportionately impacts critical outcomes (−6.5 points). A dual strategy—mandatory core-zone evacuation plus outer-zone shelter-in-place—is recommended. A resilience assessment reveals asymmetric system performance: high logistics resilience (100% supply completion) versus constrained evacuation capacity, providing insights for resource pre-positioning and fleet sizing.

Keywords: Emergency logistics; Truck-drone coordination; Multi-type evacuation; Hazardous material incident; Urban expressway; Time-space network optimization

|  | Liang Wang is a Senior Engineer at the Digital Technology Planning Center, Beijing Institute of City Planning and Design. He received his degree in Computational Mathematics from the School of Mathematical Sciences, Beijing Normal University. His research focuses on data-driven urban planning, encompassing big data analytics, urban growth modeling, urban computing, and geographic information science. Over the past five years, he has served as a core member on several major national and international research projects, including the National Key R&D Program "Theory of Smart City System Models and Simulation Computing Platform," the National Key R&D Program "Mathematical Theory and Algorithms for Key Technologies in Smart City Transportation Systems," and the World Bank-funded project "Formulation and Implementation of Transit-Oriented Development (TOD) Strategies at the City Level." He has received a First Prize in Geographic Information Science and Technology Progress, two Second Prizes and two Third Prizes in the Beijing Outstanding Planning and Design Awards, and has published over ten peer-reviewed journal papers. He also serves as an industry supervisor for undergraduate thesis projects at The Chinese University of Hong Kong, Shenzhen, and co-organizes the "Transportation System Digital Twin" workshop at the annual conference of the Digital Twin and Experimental Design Branch of the Chinese Association of On-site Statistics. |
| --- | --- |

Acknowledgements

This research was supported by the National Key Research and Development Program of China (Grant Numbers: 2021YFA1000300, 2021YFA1000303).